How to separate phases for CINE sequences:

- Go to the Advanced Viewing page (the “pumpkin” button on upper left)

- Choose the cine sequence from the thumbnails

- Right click on the thumbnail, and choose “IMAGE ALGEBRA”

- The image will open on the right ride of the screen in all 4 boxes

- Start with the upper left box:

  - You’ll see 4 grey bars. These are actually two sets of bars, broken down into “Phases” and then “Types”.

  - Using the “Types” set:

    - Slide both bars all the way to the right to choose the 3rd phase (the gray one). It should be the same in all three windows.

  - Next, using the “Phases” set”:

click on the blue icon to the right of the “A” button and the “B” button (it looks like stacked files)

  - The bar will turn solid grey, and all of the phases will be selected.

- Move your mouse to the menu bar directly above the image window.

  - From the drop down menu, choose “Addition: (A+B)”.

  - Click on the “star man” icon to the left to generate series.